Case-Study 1

Nonlinear Sensor Calibration Using Bisection Method

1. Aim

To calibrate a nonlinear flow sensor by determining the nonlinear exponent busing the Bisection Method and to compute the scaling factor A.

2. Objective

- To model a nonlinear sensor using a power-law equation.

- To formulate a nonlinear equation for calibration.

- To solve the nonlinear equation using the Bisection Method.

- To compute the scaling constant after estimating the exponent.

3. Theory

Many real sensors are nonlinear. The output voltage of the sensor does not vary linearly with flow rate. A commonly used nonlinear model is the power-law model.

The sensor model is:

Q=A(V-c)b

Where:

Q= Actual flow rate (L/s)
V= Sensor output voltage (V)
c= Baseline voltage at zero flow (V)
A= Scaling constant
b= Nonlinear exponent

4. Mathematical Derivation

Step 1: Volume as Integral of Flow

Flow rate is defined as the rate of change of volume:

Q=dVoldt

Total volume is obtained by integrating flow:

Vol=∫Q dt

Step 2: Substitute Sensor Model

Vol=∫A(V-c)b dt

Since Ais constant:

Vol=A∫(V-c)b dt

Step 3: Define Integral Term

Let:

I(b)=∫(V-c)b dt

Therefore:

Vol=A I(b)

Step 4: For Multiple Trials

Five experiments are conducted using a 3-liter syringe at different speeds.

For each trial:

I1(b)=∫(V1-c)b dt
I2(b)=∫(V2-c)b dt
I3(b)=∫(V3-c)b dt
I4(b)=∫(V4-c)b dt
I5(b)=∫(V5-c)b dt

Since the syringe volume is fixed at 3 L:

Vol1=Vol2=Vol3=Vol4=Vol5=3

Thus:

AI1(b)=AI2(b)=AI3(b)=AI4(b)=AI5(b)

Which implies:

I1(b)=I2(b)=I3(b)=I4(b)=I5(b)

Step 5: Objective Function

To make all integrals equal, define an objective function:

f(b)=stdI1(b),I2(b),I3(b),I4(b),I5(b)

For perfect calibration:

f(b)=0

This is a nonlinear equation in b.

5. Numerical Solution Using Bisection Method

To solve:

f(b)=0

We use the Bisection Method.

Bisection Equations

Initial bounds:

bl=0.5
bu=3.0

Midpoint:

br=bl+bu2

Stopping condition:

∣bu-bl∣<0.001

The iteration continues until convergence.

6. Calculation of Scaling Factor

After obtaining optimal b:

Since syringe volume is 3 L:

3=AI1(b)

Therefore:

A=3I1(b)

7. MATLAB code for iterations

clc; close all; clear all;

fs = 100;

dt = 1/fs;

c = 0.5;

Vsyringe = 3;

T = [2 3 4 5 6];

n = 5;

for j = 1:n

t = 0:dt:T(j);

for i = 1:length(t)

V(j,i) = c + (1.6-0.2*j)*exp(-t(i));

end

L(j) = length(t);

end

ml = 0.5;

mu = 3;

err = 0.001;

iter = 1;

fprintf('\nIter\tb_mid\tf(b)\tError\n');

while abs(mu-ml) > err

mr = (ml+mu)/2;

for j = 1:n

I(j) = 0;

for i = 1:L(j)

I(j) = I(j) + (V(j,i)-c)^mr * dt;

end

end

meanI = sum(I)/n;

f = sqrt(sum((I-meanI).^2)/n);

error_list(iter) = abs(mu-ml);

b_list(iter) = mr;

if f > 0

mu = mr;

else

ml = mr;

end

fprintf('%d\t%.4f\t%.6f\t%.6f\n',iter,mr,f,abs(mu-ml));

iter = iter + 1;

end

b_final = mr;

A = Vsyringe / I(1);

fprintf('\nFinal b = %.6f\n',b_final);

fprintf('Scaling A = %.6f\n',A);

b_vals = 0.5:0.05:3;

for k = 1:length(b_vals)

for j = 1:n

Itemp(j) = 0;

for i = 1:L(j)

Itemp(j) = Itemp(j) + (V(j,i)-c)^b_vals(k)*dt;

end

end

mtemp = sum(Itemp)/n;

f_vals(k) = sqrt(sum((Itemp-mtemp).^2)/n);

end

figure;

plot(b_vals,f_vals,'b','LineWidth',2);

hold on;

plot(b_final,0,'ro','MarkerSize',8,'LineWidth',2);

xlabel('Exponent b');

ylabel('f(b)');

title('Objective Function vs Exponent b');

legend('f(b)','Intersection (Optimal b)','Location','best');

grid on;

figure;

plot(1:length(error_list),error_list,'r','LineWidth',2);

hold on;

final_iter = length(error_list);

final_error = error_list(end);

plot(final_iter,final_error,'bo','MarkerSize',8,'LineWidth',2);

yline(err,'k--','Tolerance');

xlabel('Iteration');

ylabel('Error |b_u - b_l|');

title('Error Convergence of Bisection Method');

legend('Error','Final Converged Point','Tolerance','Location','best');

grid on;

OUTPUT: -

8. Final Calibrated Model

Q=A(V-c)b

Where:

bis obtained using Bisection Method
Ais computed using syringe volume

9. Result

The nonlinear exponent bwas determined using the Bisection Method.
The scaling factor Awas calculated using the known syringe volume.
The calibrated sensor equation was obtained.

10. Conclusion

The nonlinear flow sensor was successfully calibrated using numerical methods.
The experiment demonstrated practical application of:

- Numerical integration

- Nonlinear equation formulation

- Bisection Method

- Parameter estimation

This experiment shows how numerical methods are used in real-world sensor calibration problems.